Task 1 — HMI & Laptop Vision Requirements

Industry-grade Software System Requirements Specification (SRS)

Project: FOG-ORCHESTRATOR 2.0 | Task 1 — HMI & Optional Laptop Computer Vision

# 1. Purpose and Scope

Task 1 delivers the operator/control-room Human–Machine Interface (HMI) for FOG-ORCHESTRATOR 2.0. The HMI is mandatory for the SIH prototype. Computer vision is optional and, if implemented, shall execute on the teammate's laptop/central workstation—not on the ESP32 or vehicle edge controller. The HMI shall visualize live fleet state, road state, fog/visibility state, safety envelopes, bottlenecks, dispatch decisions, communication health, and alerts, while remaining clearly separated from the safety-critical vehicle controller.

# 2. System Boundary

| In Scope | Out of Scope |
| --- | --- |
| Control-room / operator HMI | Real mine FMS certification |
| Digital-twin data visualization | Direct OEM HEMM drive-by-wire certification |
| Fleet status and road-state visualization | Autonomous steering |
| Safe speed / headway / risk display | Safety-certified alarm hardware |
| Dispatch / slot command visualization and acknowledgment | On-device neural inference |
| Optional laptop-side computer vision | Field-deployed production system |
| Data logging, playback and event audit | Replacement of NMDC FMS |

# 3. Operational Concept

HMI is the human-facing supervisory layer. It does not replace the Tier-1 local physics safety governor.

The digital system may recommend or issue target speed, departure slot, route, or hold commands; the vehicle controller must enforce local safety limits.

When central communication is unavailable, HMI shall indicate degraded mode; the vehicle shall continue local-safe operation.

Optional computer vision is an information source for visibility/obstacle context and shall not directly override physics constraints.

# 4. User Roles

| Role | Primary Need | HMI Permissions |
| --- | --- | --- |
| Vehicle operator | Immediate safe speed, headway, risk, slot/route guidance | View vehicle page; acknowledge warnings |
| Control-room operator | Fleet-wide state, bottlenecks, dispatch, incidents | Fleet/road view; dispatch recommendations; acknowledge/override non-safety advisory actions |
| Safety/engineering reviewer | Traceability, event history, replay, KPIs | Read-only analytics, logs, scenario playback |
| Developer/tester | Telemetry debugging, health and data quality | Diagnostic panels; simulator controls |

# 5. Functional Requirements

| ID | Requirement | Statement |
| --- | --- | --- |
| HMI-FR-001 | Live system overview | HMI shall show current system mode, timestamp, connectivity status, active alerts, and fleet count. |
| HMI-FR-002 | Mine/road map | HMI shall render the graph-based digital mine map with road segments, grades, intersections, switchbacks, shovels, crushers and vehicle positions. |
| HMI-FR-003 | Vehicle state | For each vehicle, show ID, position/segment, speed, safe speed, headway, risk state, friction estimate (with confidence), and communication state. |
| HMI-FR-004 | Safety envelope | HMI shall clearly distinguish actual speed from v_safe and identify the active limiting constraint (visibility, friction, grade, braking/retarder, curve or site limit). |
| HMI-FR-005 | Headway monitoring | HMI shall show current headway and required H_safe for selected vehicle pairs; violation status shall be visible. |
| HMI-FR-006 | Bottleneck visualization | HMI shall rank and visually mark bottlenecks using BottleneckScore and/or utilization, queue length and criticality. |
| HMI-FR-007 | Queue monitoring | HMI shall display crusher/shovel/intersection queues, queue limits, arrival rate and service rate where available. |
| HMI-FR-008 | Arrival-rate shaping | HMI shall show planned vs actual arrivals at critical nodes and any active metering/holding decision. |
| HMI-FR-009 | Slot reservation | HMI shall show switchback/intersection reservations, ETA, slot window, current holder, and conflict status. |
| HMI-FR-010 | Dispatch decision | HMI shall show recommended/issued vehicle assignment, route and departure slot with reason code. |
| HMI-FR-011 | Fog/visibility | HMI shall show current visibility and, when available, forecast visibility with uncertainty/confidence band. |
| HMI-FR-012 | Road condition | HMI shall show friction estimate, road wetness/surface state and optional roughness state where available. |
| HMI-FR-013 | Communication health | HMI shall show V2V/V2I/LoRa link state, packet freshness/age and degraded-mode state. |
| HMI-FR-014 | Alerts | HMI shall generate prioritized safety/operational alerts including unsafe speed, unsafe headway, bottleneck risk, communication loss, stale data and slot conflict. |
| HMI-FR-015 | Alert acknowledgment | Operators shall be able to acknowledge applicable advisory/operational alerts; safety-critical local protection shall not depend on acknowledgment. |
| HMI-FR-016 | System modes | HMI shall show NORMAL, CAUTION, DEGRADED, LOCAL-SAFE and STOP/UNSAFE modes. |
| HMI-FR-017 | Replay | HMI shall support time-based replay of logged simulation/HIL telemetry for post-event analysis. |
| HMI-FR-018 | KPI dashboard | HMI shall show throughput, cycle time, queue length, utilization, stops, safety-envelope violations and recovery time. |
| HMI-FR-019 | Scenario controls | Developer/test view shall permit loading named fog, friction, grade, fleet-density and communication-loss scenarios. |
| HMI-FR-020 | CV integration (optional) | If CV is implemented, HMI shall display camera frame, visibility estimate/confidence and detected obstacle/road-state overlays without placing inference on the ESP32. |

# 6. Non-Functional Requirements

| ID | Category | Requirement |
| --- | --- | --- |
| HMI-NFR-001 | Performance | Primary dashboard update target: ≤1 s for live state refresh under prototype load. |
| HMI-NFR-002 | Command latency | Display of a new central decision target: ≤2 s from receipt under nominal LAN conditions; measured separately from vehicle actuation latency. |
| HMI-NFR-003 | Data freshness | Each live datum shall carry timestamp/source age; stale data shall be visually flagged after configurable timeout. |
| HMI-NFR-004 | Availability | HMI should recover automatically after temporary backend/reconnect interruption without corrupting state. |
| HMI-NFR-005 | Safety separation | Loss or crash of HMI shall not disable Tier-1 vehicle safety. |
| HMI-NFR-006 | Explainability | Every dispatch/risk decision shown to the operator shall include a concise reason code and key limiting variables. |
| HMI-NFR-007 | Traceability | All issued/received commands and mode transitions shall be timestamped and logged. |
| HMI-NFR-008 | Usability | Critical information shall be readable at a glance; avoid relying on color alone for safety state. |
| HMI-NFR-009 | Scalability | Prototype HMI shall support at least 50 simulated vehicles and 20 physical/logical road nodes. |
| HMI-NFR-010 | Portability | Laptop deployment shall work from documented setup steps and use open-source tooling where practical. |
| HMI-NFR-011 | Security baseline | Prototype communications shall include authentication/configurable access controls where supported; no hard-coded credentials in source. |
| HMI-NFR-012 | Recovery | If backend data stops, the HMI shall enter DATA-STale/DEGRADED indication rather than freezing values silently. |

# 7. Minimum HMI Screens

| Screen | Mandatory Content |
| --- | --- |
| S1 — Operations Overview | Mine map, vehicles, active bottleneck, current fog state, top alerts, throughput and system mode. |
| S2 — Vehicle Detail | Speed, safe speed, headway, risk, friction, grade, communication, active constraint, recent trend. |
| S3 — Bottleneck & Queue | Crusher/shovel/intersection utilization, arrival/service rates, queue lengths, predicted queue trend. |
| S4 — Dispatch & Slots | Truck assignment, departure time, route, switchback/intersection slot timeline, reason codes. |
| S5 — Event/Replay | Timeline of fog, alerts, commands, queue changes, violations and recovery. |
| S6 — Diagnostics | Data freshness, sensor/telemetry status, backend health, message counts, latency. |

# 8. Optional Laptop Computer Vision

Computer vision is optional for Task 1 and runs on the teammate laptop/central workstation. It shall be treated as a non-safety-critical perception aid. It may provide: visibility estimation from contrast/haze metrics; obstacle/vehicle detection; road wetness/surface classification; and visual overlays. The CV subsystem shall publish a timestamped estimate plus confidence. The physics/safety engine remains authoritative.

| CV ID | Requirement | Acceptance |
| --- | --- | --- |
| CV-001 | Camera ingestion | Read live webcam/IP stream or recorded video. |
| CV-002 | Visibility estimate | Produce an interpretable visibility/scene-visibility proxy plus confidence. |
| CV-003 | Obstacle/vehicle detection | Optional detector shall identify configured classes in test footage. |
| CV-004 | Latency | Target ≤200 ms from frame capture to published CV result on test laptop; report measured value. |
| CV-005 | Graceful failure | Camera failure shall set CV status to unavailable; no stale output may be silently treated as current. |
| CV-006 | Integration | Publish results to HMI/backend with timestamp, source ID and confidence. |

# 9. Data Interfaces

| Message | Minimum Fields |
| --- | --- |
| VehicleState | vehicle_id, timestamp, x/y or segment_id, speed_mps, accel_mps2, grade_rad, friction_est, friction_sigma, mode, comm_confidence |
| SafetyState | vehicle_id, v_safe, h_safe, actual_speed, active_constraint, risk_level |
| RoadState | segment_id, visibility_m, visibility_sigma, friction, grade, capacity_vph, queue, utilization |
| BottleneckState | node_id, lambda_vph, mu_vph, queue, queue_max, criticality, bottleneck_score |
| DispatchCommand | vehicle_id, route_id, departure_time, target_speed, slot_id, reason_code, timestamp, command_id |
| SlotState | slot_id, resource_id, vehicle_id, start_time, end_time, status |
| CVResult | timestamp, source_id, visibility_proxy, confidence, detections[], road_state |
| Health | component_id, timestamp, state, latency_ms, age_ms, error_code |

# 10. Technology Baseline

Prototype frontend: web-based HMI is preferred for fast demonstration and platform portability (e.g., React/HTML/JS or Streamlit if time-constrained).

Backend/API: Python service (e.g., FastAPI/Flask) or direct integration with the digital-twin process.

Real-time transport: WebSocket/MQTT/local message bus; final choice shall be documented.

Charts: Plotly/Matplotlib-compatible exported data is acceptable.

Computer vision: laptop-side Python/OpenCV and an approved detector if used; no ESP32 inference required.

# 11. Requirement Traceability Principle

Every HMI function shall have a unique requirement ID, an implementation reference, and at least one verification method. Requirements marked mandatory must not be waived without team-lead approval. Optional CV requirements can be marked N/A without affecting HMI completion.

# 12. Task 1 Completion Definition

Mandatory HMI screens implemented and connected to live/simulated data.

All mandatory HMI functional requirements verified.

Stale-data and communication-loss behavior demonstrated.

Safety-critical values (v_safe, h_safe, risk, active constraint) visible and traceable.

Bottleneck, queue, arrival shaping and slot reservation visible.

Logs and replay available.

Optional CV may be separately marked Complete / Partial / Not Implemented.